Build Your Own Desktop PC – Part 1

Basic Computer Components

Computer Case – Houses all the computer components to form the tower

- Various sizes – typical size is a mid or full tower

- Price Range: $20 - $350

CPU/APU - The main chip that processes programs and apps

- CPU - Central Processing Unit

- APU – Accelerated Processing Unit (includes graphics)

- Use www.passmark.com to determine a component’s score

|  | Light User | Moderate User | Power User |
| --- | --- | --- | --- |
| Passmark Score | APU: 3000+ | APU/CPU: 5000+ | APU/CPU: 8000+ |
| Price Range | $55-$100 | $110-$180 | $200-$1000 |

Motherboard - The mainboard that connects all your components

- Match your motherboard to your CPU (brand, socket, case)

- Consider features (Ram support, Sata slots, Video support, ports, etc.)

- Price Range: $35-$$350

GPU (Graphics Processing Unit) – Allows images to be displayed on the screen

|  | Light User | Moderate User | Power User |
| --- | --- | --- | --- |
| Passmark Score | APU can handle graphics | GPU: 4000+ | GPU: 7000+ |
| Price Range | $0 | $120-$200 | $250-$1000 |

RAM (Random Access Memory) – Runs all of your programs and apps

- Size: Minimum – 4GB, Recommended – 8GB+

- Must be compatible with your motherboard

- DDR4 is the new standard, making it faster and more expensive

- Price Range: $5-$10 per GB

Hard Drive - Saves your files, programs, and apps to the computer

- SSD drives – Quickest

- Hybrid drives – Quick with high storage

- Mechanical hard drive – Most common, slowest w/ highest storage

|  | Low Storage User (SSD) | High Storage User |
| --- | --- | --- |
| Capacity | 120GB-240 GB | 500GB-2TB |
| Price Range | $45-$120 | $40-$150 |

Optical Drive – For reading discs (CD, DVD, Blu-Ray)

- If you will not load media or watch videos from discs, an optical drive is not necessary

- Price Range: $20 - $200

Cooling Fans - Keeps the parts cool to prevent overheating/damage

- 1 fan is typical for light user; 2+ for moderate & power users

- Quieter fans begin in the 17dB level and lower

- Common sizes: 120mm or 92mm

- Price Range: $6-$40

PSU (Power Supply Unit) – Powers all of your components

- Use a PSU calculator to determine wattage needs

  - http://outervision.com/power-supply-calculator

- Verify your 12-volt rail has enough Amps for your graphics card

- Price Range: $40-$200

Operating Systems

Windows

- Windows 10 is the current version

- Beware of when security updates will end for older versions (ex: Windows 7 ends in 2020)

Linux

- Free alternative to Windows but geared towards tech-savy crowd

- Umbuntu, Mint, Lumbuntu are the popular versions

Trusted Brands

Less Reliable Brands

| ASUS | Adata | Patriot |
| --- | --- | --- |
| Gigabyte | Western Digital | ECS Elite Group |
| MSI | EVGA | Biostar |
| Kingston | XFX | Logisys |
| Corsair | Zotac | Super Talent |

Consider purchasing components in person at Fry’s Electronics, Micro Center or online through Newegg or Amazon